例程20 1602液晶实验
本次试验使用arduino直接驱动1602液晶显示文字

1602液晶在应用中非常广泛，最初的1602液晶使用的是HD44780控制器，现在各个厂家的1602模块基本上都是采用了与之兼容的IC，所以特性上基本都是一致的。

1602LCD主要技术参数
显示容量为16×2个字符；
芯片工作电压为4.5～5.5V；
工作电流为2.0mA（5.0V）；
模块最佳工作电压为5.0V；
字符尺寸为2.95×4.35（W×H）mm。

1602液晶接口引脚定义


 
接口说明：
1、两组电源 一组是模块的电源 一组是背光板的电源 一般均使用5V供电。本次试验背光使用3.3V供电也可以工作。
2、VL是调节对比度的引脚，串联不大于5KΩ的电位器进行调节。本次实验使用1KΩ的电阻来设定对比度。其连接分高电位与低电位接法，本次使用低电位接法，串联1KΩ电阻后接GND。
3、RS 是很多液晶上都有的引脚 是命令/数据选择引脚 该脚电平为高时表示将进行数据操作；为低时表示进行命令操作。
4、RW 也是很多液晶上都有的引脚 是读写选择端 该脚电平为高是表示要对液晶进行读操作；为低时表示要进行写操作。
5、E 同样很多液晶模块有此引脚 通常在总线上信号稳定后给一正脉冲通知把数据读走，在此脚为高电平的时候总线不允许变化。
6、D0—D7 8 位双向并行总线，用来传送命令和数据。
7、BLA是背光源正极，BLK是背光源负极。

1602液晶的基本操作分以下四种：

[image: image1.jpg]» R/W=H,

E=H

DO~D7=H#EF

» RIW=L,

DO~D7=15473, E=&fikH

T

» R/W=H,

E=H

[DO~D7=4&

» RIW=L,

DO~D7=#1%, E=&fkH

T

From www.geek-workshop.






下图就是1602液晶实物图

[image: image2.jpg]E
g
=
E
H
e
H
[
£
e
g






[image: image3.jpg]





1602直接与arduino通信，根据产品手册描述，分8位连接法与4位连接法，咱们先使用8位连接法进行实验。硬件连接方式如下图

[image: image4.jpg]




[image: image5.jpg]5
g
2
K
5
H
o
g
E
o
£







代码如下 

int DI = 12;

int RW = 11;

int DB[] = {3, 4, 5, 6, 7, 8, 9, 10};//使用数组来定义总线需要的管脚

int Enable = 2;

void LcdCommandWrite(int value) {

// 定义所有引脚

int i = 0;

for (i=DB[0]; i <= DI; i++) //总线赋值

{

   digitalWrite(i,value & 01);//因为1602液晶信号识别是D7-D0(不是D0-D7)，这里是用来反转信号。

   value >>= 1;

}

digitalWrite(Enable,LOW);

delayMicroseconds(1);

digitalWrite(Enable,HIGH);

delayMicroseconds(1);  // 延时1ms

digitalWrite(Enable,LOW);

delayMicroseconds(1);  // 延时1ms

}

void LcdDataWrite(int value) {

// 定义所有引脚

int i = 0;

digitalWrite(DI, HIGH);

digitalWrite(RW, LOW);

for (i=DB[0]; i <= DB[7]; i++) {

   digitalWrite(i,value & 01);

   value >>= 1;

}

digitalWrite(Enable,LOW);

delayMicroseconds(1);

digitalWrite(Enable,HIGH);

delayMicroseconds(1);

digitalWrite(Enable,LOW);

delayMicroseconds(1);  // 延时1ms

}

void setup (void) {

int i = 0;

for (i=Enable; i <= DI; i++) {

   pinMode(i,OUTPUT);

}

delay(100);

// 短暂的停顿后初始化LCD

// 用于LCD控制需要

LcdCommandWrite(0x38);  // 设置为8-bit接口，2行显示，5x7文字大小                     

delay(64);                      

LcdCommandWrite(0x38);  // 设置为8-bit接口，2行显示，5x7文字大小                        

delay(50);                      

LcdCommandWrite(0x38);  // 设置为8-bit接口，2行显示，5x7文字大小                        

delay(20);                      

LcdCommandWrite(0x06);  // 输入方式设定

                         // 自动增量，没有显示移位

delay(20);                      

LcdCommandWrite(0x0E);  // 显示设置

                         // 开启显示屏，光标显示，无闪烁

delay(20);                      

LcdCommandWrite(0x01);  // 屏幕清空，光标位置归零  

delay(100);                      

LcdCommandWrite(0x80);  // 显示设置

                         // 开启显示屏，光标显示，无闪烁

delay(20);                      

}

void loop (void) {

  LcdCommandWrite(0x01);  // 屏幕清空，光标位置归零  

  delay(10); 

  LcdCommandWrite(0x80+3); 

  delay(10);                     

  // 写入欢迎信息 

  LcdDataWrite('W');

  LcdDataWrite('e');

  LcdDataWrite('l');

  LcdDataWrite('c');

  LcdDataWrite('o');

  LcdDataWrite('m');

  LcdDataWrite('e');

  LcdDataWrite(' ');

  LcdDataWrite('t');

  LcdDataWrite('o');

  delay(10);

  LcdCommandWrite(0xc0+1);  // 定义光标位置为第二行第二个位置  

  delay(10); 

  LcdDataWrite('g');

  LcdDataWrite('e');

  LcdDataWrite('e');

  LcdDataWrite('k');

  LcdDataWrite('-');

  LcdDataWrite('w');

  LcdDataWrite('o');

  LcdDataWrite('r');

  LcdDataWrite('k');

  LcdDataWrite('s');

  LcdDataWrite('h');

  LcdDataWrite('o');

  LcdDataWrite('p');

  delay(5000);

  LcdCommandWrite(0x01);  // 屏幕清空，光标位置归零  

  delay(10);

  LcdDataWrite('I');

  LcdDataWrite(' ');

  LcdDataWrite('a');

  LcdDataWrite('m');

  LcdDataWrite(' ');

  LcdDataWrite('h');

  LcdDataWrite('o');

  LcdDataWrite('n');

  LcdDataWrite('g');

  LcdDataWrite('y');

  LcdDataWrite('i');

  delay(3000);

  LcdCommandWrite(0x02); //设置模式为新文字替换老文字，无新文字的地方显示不变。

  delay(10);

  LcdCommandWrite(0x80+5); //定义光标位置为第一行第六个位置

  delay(10);  

  LcdDataWrite('t');

  LcdDataWrite('h');

  LcdDataWrite('e');

  LcdDataWrite(' ');

  LcdDataWrite('a');

  LcdDataWrite('d');

  LcdDataWrite('m');

  LcdDataWrite('i');

  LcdDataWrite('n');

  delay(5000);

}

[image: image6.png]


int DI = 12;

[image: image7.png]


int RW = 11;

[image: image8.png]


int DB[] = {3, 4, 5, 6, 7, 8, 9, 10};//使用数组来定义总线需要的管脚

[image: image9.png]


int Enable = 2;

[image: image10.png]



[image: image11.png]


void LcdCommandWrite(int value) {

[image: image12.png]


 // 定义所有引脚

[image: image13.png]


 int i = 0;

[image: image14.png]


 for (i=DB[0]; i <= DI; i++) //总线赋值

[image: image15.png]


{

[image: image16.png]


   digitalWrite(i,value & 01);//因为1602液晶信号识别是D7-D0(不是D0-D7)，这里是用来反转信号。

[image: image17.png]


   value >>= 1;

[image: image18.png]


 }

[image: image19.png]


 digitalWrite(Enable,LOW);

[image: image20.png]


 delayMicroseconds(1);

[image: image21.png]


 digitalWrite(Enable,HIGH);

[image: image22.png]


 delayMicroseconds(1);  // 延时1ms

[image: image23.png]


 digitalWrite(Enable,LOW);

[image: image24.png]


 delayMicroseconds(1);  // 延时1ms

[image: image25.png]


}

[image: image26.png]



[image: image27.png]


void LcdDataWrite(int value) {

[image: image28.png]


 // 定义所有引脚

[image: image29.png]


 int i = 0;

[image: image30.png]


 digitalWrite(DI, HIGH);

[image: image31.png]


 digitalWrite(RW, LOW);

[image: image32.png]


 for (i=DB[0]; i <= DB[7]; i++) {

[image: image33.png]


   digitalWrite(i,value & 01);

[image: image34.png]


   value >>= 1;

[image: image35.png]


 }

[image: image36.png]


 digitalWrite(Enable,LOW);

[image: image37.png]


 delayMicroseconds(1);

[image: image38.png]


 digitalWrite(Enable,HIGH);

[image: image39.png]


 delayMicroseconds(1);

[image: image40.png]


 digitalWrite(Enable,LOW);

[image: image41.png]


 delayMicroseconds(1);  // 延时1ms

[image: image42.png]


}

[image: image43.png]



[image: image44.png]


void setup (void) {

[image: image45.png]


 int i = 0;

[image: image46.png]


 for (i=Enable; i <= DI; i++) {

[image: image47.png]


   pinMode(i,OUTPUT);

[image: image48.png]


 }

[image: image49.png]


 delay(100);

[image: image50.png]


 // 短暂的停顿后初始化LCD

[image: image51.png]


 // 用于LCD控制需要

[image: image52.png]


 LcdCommandWrite(0x38);  // 设置为8-bit接口，2行显示，5x7文字大小                     

[image: image53.png]


 delay(64);                      

[image: image54.png]


 LcdCommandWrite(0x38);  // 设置为8-bit接口，2行显示，5x7文字大小                        

[image: image55.png]


 delay(50);                      

[image: image56.png]


 LcdCommandWrite(0x38);  // 设置为8-bit接口，2行显示，5x7文字大小                        

[image: image57.png]


 delay(20);                      

[image: image58.png]


 LcdCommandWrite(0x06);  // 输入方式设定

[image: image59.png]


                         // 自动增量，没有显示移位

[image: image60.png]


 delay(20);                      

[image: image61.png]


 LcdCommandWrite(0x0E);  // 显示设置

[image: image62.png]


                         // 开启显示屏，光标显示，无闪烁

[image: image63.png]


 delay(20);                      

[image: image64.png]


 LcdCommandWrite(0x01);  // 屏幕清空，光标位置归零  

[image: image65.png]


 delay(100);                      

[image: image66.png]


 LcdCommandWrite(0x80);  // 显示设置

[image: image67.png]


                         // 开启显示屏，光标显示，无闪烁

[image: image68.png]


 delay(20);                      

[image: image69.png]


}

[image: image70.png]



[image: image71.png]


void loop (void) {

[image: image72.png]


  LcdCommandWrite(0x01);  // 屏幕清空，光标位置归零  

[image: image73.png]


  delay(10); 

[image: image74.png]


  LcdCommandWrite(0x80+3); 

[image: image75.png]


  delay(10);                     

[image: image76.png]


  // 写入欢迎信息 

[image: image77.png]


  LcdDataWrite('W');

[image: image78.png]


  LcdDataWrite('e');

[image: image79.png]


  LcdDataWrite('l');

[image: image80.png]


  LcdDataWrite('c');

[image: image81.png]


  LcdDataWrite('o');

[image: image82.png]


  LcdDataWrite('m');

[image: image83.png]


  LcdDataWrite('e');

[image: image84.png]


  LcdDataWrite(' ');

[image: image85.png]


  LcdDataWrite('t');

[image: image86.png]


  LcdDataWrite('o');

[image: image87.png]


  delay(10);

[image: image88.png]


  LcdCommandWrite(0xc0+1);  // 定义光标位置为第二行第二个位置  

[image: image89.png]


  delay(10); 

[image: image90.png]


  LcdDataWrite('g');

[image: image91.png]


  LcdDataWrite('e');

[image: image92.png]


  LcdDataWrite('e');

[image: image93.png]


  LcdDataWrite('k');

[image: image94.png]


  LcdDataWrite('-');

[image: image95.png]


  LcdDataWrite('w');

[image: image96.png]


  LcdDataWrite('o');

[image: image97.png]


  LcdDataWrite('r');

[image: image98.png]


  LcdDataWrite('k');

[image: image99.png]


  LcdDataWrite('s');

[image: image100.png]


  LcdDataWrite('h');

[image: image101.png]


  LcdDataWrite('o');

[image: image102.png]


  LcdDataWrite('p');

[image: image103.png]


  delay(5000);

[image: image104.png]


  LcdCommandWrite(0x01);  // 屏幕清空，光标位置归零  

[image: image105.png]


  delay(10);

[image: image106.png]


  LcdDataWrite('I');

[image: image107.png]


  LcdDataWrite(' ');

[image: image108.png]


  LcdDataWrite('a');

[image: image109.png]


  LcdDataWrite('m');

[image: image110.png]


  LcdDataWrite(' ');

[image: image111.png]


  LcdDataWrite('h');

[image: image112.png]


  LcdDataWrite('o');

[image: image113.png]


  LcdDataWrite('n');

[image: image114.png]


  LcdDataWrite('g');

[image: image115.png]


  LcdDataWrite('y');

[image: image116.png]


  LcdDataWrite('i');

[image: image117.png]


  delay(3000);

[image: image118.png]


  LcdCommandWrite(0x02); //设置模式为新文字替换老文字，无新文字的地方显示不变。

[image: image119.png]


  delay(10);

[image: image120.png]


  LcdCommandWrite(0x80+5); //定义光标位置为第一行第六个位置

[image: image121.png]


  delay(10);  

[image: image122.png]


  LcdDataWrite('t');

[image: image123.png]


  LcdDataWrite('h');

[image: image124.png]


  LcdDataWrite('e');

[image: image125.png]


  LcdDataWrite(' ');

[image: image126.png]


  LcdDataWrite('a');

[image: image127.png]


  LcdDataWrite('d');

[image: image128.png]


  LcdDataWrite('m');

[image: image129.png]


  LcdDataWrite('i');

[image: image130.png]


  LcdDataWrite('n');

[image: image131.png]


  delay(5000);

[image: image132.png]


}

4位接法

在正常使用下，8位接法基本把arduino的数字端口占满了，如果想要多接几个传感器就没有端口了，这种情况下怎么处理呢，咱们可以使用4位接法。

4位接法的硬件连接方法如下图

[image: image133.jpg]




[image: image134.jpg]





硬件接好后把下面的代码上传到控制板上，看看效果。 

int LCD1602_RS=12;   

int LCD1602_RW=11;   

int LCD1602_EN=10;   

int DB[] = { 6, 7, 8, 9};

char str1[]="Welcome to";

char str2[]="geek-workshop";

char str3[]="this is the";

char str4[]="4-bit interface";

void LCD_Command_Write(int command)

{

int i,temp;

digitalWrite( LCD1602_RS,LOW);

digitalWrite( LCD1602_RW,LOW);

digitalWrite( LCD1602_EN,LOW);

temp=command & 0xf0;

for (i=DB[0]; i <= 9; i++)

{

   digitalWrite(i,temp & 0x80);

   temp <<= 1;

}

digitalWrite( LCD1602_EN,HIGH);

delayMicroseconds(1);

digitalWrite( LCD1602_EN,LOW);

temp=(command & 0x0f)<<4;

for (i=DB[0]; i <= 10; i++)

{

   digitalWrite(i,temp & 0x80);

   temp <<= 1;

}

digitalWrite( LCD1602_EN,HIGH);

delayMicroseconds(1); 

digitalWrite( LCD1602_EN,LOW);

}

void LCD_Data_Write(int dat)

{

int i=0,temp;

digitalWrite( LCD1602_RS,HIGH);

digitalWrite( LCD1602_RW,LOW);

digitalWrite( LCD1602_EN,LOW);

temp=dat & 0xf0;

for (i=DB[0]; i <= 9; i++)

{

   digitalWrite(i,temp & 0x80);

   temp <<= 1;

}

digitalWrite( LCD1602_EN,HIGH);

delayMicroseconds(1);

digitalWrite( LCD1602_EN,LOW);

temp=(dat & 0x0f)<<4;

for (i=DB[0]; i <= 10; i++)

{

   digitalWrite(i,temp & 0x80);

   temp <<= 1;

}

digitalWrite( LCD1602_EN,HIGH);

delayMicroseconds(1); 

digitalWrite( LCD1602_EN,LOW);

}

void LCD_SET_XY( int x, int y )

{

  int address;

  if (y ==0)    address = 0x80 + x;

  else          address = 0xC0 + x;

  LCD_Command_Write(address); 

}

void LCD_Write_Char( int x,int y,int dat)

{

  LCD_SET_XY( x, y ); 

  LCD_Data_Write(dat);

}

void LCD_Write_String(int X,int Y,char *s)

{

    LCD_SET_XY( X, Y );    //设置地址 

    while (*s)             //写字符串

    {

      LCD_Data_Write(*s);   

      s ++;

    }

}

void setup (void) 

{

  int i = 0;

  for (i=6; i <= 12; i++) 

   {

     pinMode(i,OUTPUT);

   }

  delay(100);

  LCD_Command_Write(0x28);//4线 2行 5x7

  delay(50); 

  LCD_Command_Write(0x06);

  delay(50); 

  LCD_Command_Write(0x0c);

  delay(50); 

  LCD_Command_Write(0x80);

  delay(50); 

  LCD_Command_Write(0x01);

  delay(50); 

}

void loop (void)

{

   LCD_Command_Write(0x01);

   delay(50);

   LCD_Write_String(3,0,str1);//第1行，第4个地址起

   delay(50);

   LCD_Write_String(1,1,str2);//第2行，第2个地址起

   delay(5000);

   LCD_Command_Write(0x01);

   delay(50);

   LCD_Write_String(0,0,str3);

   delay(50);

   LCD_Write_String(0,1,str4);

   delay(5000);

   

}

普通浏览复制代码保存代码打印代码

[image: image135.png]


int LCD1602_RS=12;   

[image: image136.png]


int LCD1602_RW=11;   

[image: image137.png]


int LCD1602_EN=10;   

[image: image138.png]


int DB[] = { 6, 7, 8, 9};

[image: image139.png]


char str1[]="Welcome to";

[image: image140.png]


char str2[]="geek-workshop";

[image: image141.png]


char str3[]="this is the";

[image: image142.png]


char str4[]="4-bit interface";

[image: image143.png]



[image: image144.png]


void LCD_Command_Write(int command)

[image: image145.png]


{

[image: image146.png]


 int i,temp;

[image: image147.png]


 digitalWrite( LCD1602_RS,LOW);

[image: image148.png]


 digitalWrite( LCD1602_RW,LOW);

[image: image149.png]


 digitalWrite( LCD1602_EN,LOW);

[image: image150.png]



[image: image151.png]


 temp=command & 0xf0;

[image: image152.png]


 for (i=DB[0]; i <= 9; i++)

[image: image153.png]


 {

[image: image154.png]


   digitalWrite(i,temp & 0x80);

[image: image155.png]


   temp <<= 1;

[image: image156.png]


 }

[image: image157.png]


 

[image: image158.png]


 digitalWrite( LCD1602_EN,HIGH);

[image: image159.png]


 delayMicroseconds(1);

[image: image160.png]


 digitalWrite( LCD1602_EN,LOW);

[image: image161.png]



[image: image162.png]


 temp=(command & 0x0f)<<4;

[image: image163.png]


 for (i=DB[0]; i <= 10; i++)

[image: image164.png]


 {

[image: image165.png]


   digitalWrite(i,temp & 0x80);

[image: image166.png]


   temp <<= 1;

[image: image167.png]


 }

[image: image168.png]



[image: image169.png]


 digitalWrite( LCD1602_EN,HIGH);

[image: image170.png]


 delayMicroseconds(1); 

[image: image171.png]


 digitalWrite( LCD1602_EN,LOW);

[image: image172.png]


}

[image: image173.png]



[image: image174.png]


void LCD_Data_Write(int dat)

[image: image175.png]


{

[image: image176.png]


 int i=0,temp;

[image: image177.png]


 digitalWrite( LCD1602_RS,HIGH);

[image: image178.png]


 digitalWrite( LCD1602_RW,LOW);

[image: image179.png]


 digitalWrite( LCD1602_EN,LOW);

[image: image180.png]



[image: image181.png]


 temp=dat & 0xf0;

[image: image182.png]


 for (i=DB[0]; i <= 9; i++)

[image: image183.png]


 {

[image: image184.png]


   digitalWrite(i,temp & 0x80);

[image: image185.png]


   temp <<= 1;

[image: image186.png]


 }

[image: image187.png]



[image: image188.png]


 digitalWrite( LCD1602_EN,HIGH);

[image: image189.png]


 delayMicroseconds(1);

[image: image190.png]


 digitalWrite( LCD1602_EN,LOW);

[image: image191.png]



[image: image192.png]


 temp=(dat & 0x0f)<<4;

[image: image193.png]


 for (i=DB[0]; i <= 10; i++)

[image: image194.png]


 {

[image: image195.png]


   digitalWrite(i,temp & 0x80);

[image: image196.png]


   temp <<= 1;

[image: image197.png]


 }

[image: image198.png]



[image: image199.png]


 digitalWrite( LCD1602_EN,HIGH);

[image: image200.png]


 delayMicroseconds(1); 

[image: image201.png]


 digitalWrite( LCD1602_EN,LOW);

[image: image202.png]


}

[image: image203.png]



[image: image204.png]


void LCD_SET_XY( int x, int y )

[image: image205.png]


{

[image: image206.png]


  int address;

[image: image207.png]


  if (y ==0)    address = 0x80 + x;

[image: image208.png]


  else          address = 0xC0 + x;

[image: image209.png]


  LCD_Command_Write(address); 

[image: image210.png]


}

[image: image211.png]



[image: image212.png]


void LCD_Write_Char( int x,int y,int dat)

[image: image213.png]


{

[image: image214.png]


  LCD_SET_XY( x, y ); 

[image: image215.png]


  LCD_Data_Write(dat);

[image: image216.png]


}

[image: image217.png]



[image: image218.png]


void LCD_Write_String(int X,int Y,char *s)

[image: image219.png]


{

[image: image220.png]


    LCD_SET_XY( X, Y );    //设置地址 

[image: image221.png]


    while (*s)             //写字符串

[image: image222.png]


    {

[image: image223.png]


      LCD_Data_Write(*s);   

[image: image224.png]


      s ++;

[image: image225.png]


    }

[image: image226.png]


}

[image: image227.png]



[image: image228.png]


void setup (void) 

[image: image229.png]


{

[image: image230.png]


  int i = 0;

[image: image231.png]


  for (i=6; i <= 12; i++) 

[image: image232.png]


   {

[image: image233.png]


     pinMode(i,OUTPUT);

[image: image234.png]


   }

[image: image235.png]


  delay(100);

[image: image236.png]


  LCD_Command_Write(0x28);//4线 2行 5x7

[image: image237.png]


  delay(50); 

[image: image238.png]


  LCD_Command_Write(0x06);

[image: image239.png]


  delay(50); 

[image: image240.png]


  LCD_Command_Write(0x0c);

[image: image241.png]


  delay(50); 

[image: image242.png]


  LCD_Command_Write(0x80);

[image: image243.png]


  delay(50); 

[image: image244.png]


  LCD_Command_Write(0x01);

[image: image245.png]


  delay(50); 

[image: image246.png]



[image: image247.png]


}

[image: image248.png]



[image: image249.png]


void loop (void)

[image: image250.png]


{

[image: image251.png]


   LCD_Command_Write(0x01);

[image: image252.png]


   delay(50);

[image: image253.png]


   LCD_Write_String(3,0,str1);//第1行，第4个地址起

[image: image254.png]


   delay(50);

[image: image255.png]


   LCD_Write_String(1,1,str2);//第2行，第2个地址起

[image: image256.png]


   delay(5000);

[image: image257.png]


   LCD_Command_Write(0x01);

[image: image258.png]


   delay(50);

[image: image259.png]


   LCD_Write_String(0,0,str3);

[image: image260.png]


   delay(50);

[image: image261.png]


   LCD_Write_String(0,1,str4);

[image: image262.png]


   delay(5000);

[image: image263.png]


   

[image: image264.png]


}

